Specifier General Notes:

The purpose of this guide specification is to specify Intentek™ Wireless Charging Surface by Formica Group for any given project to be included in the project manual or sheet specification. This too could serve as text to "drop-into" laminated related specification sections.

Formica® Brand products support conservation, green design and building practices and may contribute toward many sustainability initiatives including LEED (Leadership in Energy and Environmental Design) building certifications.

Consult Formica Corporation, http://www.formica.com/en/us for additional assistance in using this information for additional information not included, laminate types, application, fabrication and the latest colors and finishes.

October 2019.

SECTION 06 05 13 – DECORATIVE PLASTIC LAMINATE FINISHES

GENERAL

SUMMARY

Section Includes the following Laminates:

Specifier Note:

Select all that apply

[Formica Group Intentek™ Wireless Charging Surface]

Related Requirements:

Specifier Note:

Select all that apply

[Division 06 Interior Finish Carpentry.]

[Division 06 Plastic-Laminate-Faced Architectural Cabinets.]

[Division 06 Interior Architectural Woodwork.]

[Division 12 Manufactured Plastic-Laminate-Faced Casework.]

[Division 12 Residential Casework.]

[Division 12 Plastic-Laminate-Clad Countertops.]

[Division 12 Plastic-Laminate-Clad Case Goods.]

[Division 12  Furniture]

REFERENCES

Reference Standards: In addition to requirements, comply with applicable provisions of following for design, materials, fabrication, and installation of component parts:

ANSI / NEMA LD-3: High Pressure Decorative Laminates.

ASTM E 84: Standard Test Method for Surface Burning Characteristics of Building Materials.

Qi Certified Specifications for Wireless Charging

UL 2738

UL 60950

Specifier Note:

Select all that apply

[ASTM E 162: Standard Test Method for Surface Flammability of Materials Using a Radiant Heat Energy Source.]

[FSC: Forest Stewardship Council.]

[IMO FTP: International Code for Application of Fire Test Procedures.]

[IMO FTP Code Part 2: Smoke and Density Test.]

[IMO FTP Code Part 5: Test for Surface Flammability.]

[ISO: International Organization for Standardization.]

[ISO 9001: Quality Management Systems.]

[ISO 14021: Environmental Management Systems.]

[ISO 4586: Specifies the methods of test for determination of the properties of High-Pressure Decorative Laminates (HPL, HPDL).]

[LEED: Leadership in Energy and Environmental Design.]

[NFPA 101: Life Safety Code.]

[NSF 35: High Pressure Decorative Laminates for Surfacing Food Service Equipment.]

[Architectural Woodwork Quality Standards.]

PREINSTALLATION MEETINGS

Preinstallation Conference:  Conduct conference at Project site.

Specifier Note:

Retain following article / paragraphs if required for any sustainable requirements. Add additional sustainable requirements not listed. Check also manufacturer's website for current information.

ACTION SUBMITTALS

Product Data:

          - Product data for each specified product. Include manufacturer’s technical data sheets and published instructions.

Shop Drawings: Each installation.

Anchorages to other construction, including requirements for concealed supports.

Use same unit designations used on Drawings.

Samples for Selection:

Available standard framing finish colors.

Verification Samples: 12-inch length of typical framing member in specified finish.

Plastic-laminate-clad panels, not less than 8 by 10 inches (200 by 250 mm), for each type, color, pattern, and surface finish[, with separate samples of unfaced panel product used for core].

Thermoset decorative-overlay-surfaced panels, not less than 8 by 10 inches (200 by 250 mm), for each type, color, pattern, and surface finish.

INFORMATIONAL SUBMITTALS

Qualification Data: For fabricator and installer.

Retain "Product Certificates" Paragraph below to require submittal of product certificates from manufacturers.

Product Certificates: For the following:

Retain and revise subparagraphs below if retaining second option in "Product Certificates" Paragraph above.

Thermoset decorative panels.

High-pressure decorative laminate.

Retain "Evaluation Reports" Paragraph below for fire-retardant-treated materials.

Evaluation Reports: For fire-retardant-treated materials, from ICC-ES.

Specifier Note:

Retain following article / paragraphs if required for any sustainable requirements. Add additional sustainable requirements not listed. Check also manufacturer's website for current information.

SUSTAINABLE DESIGN SUBMITTALS

        - General:  Products in this section may also be required to comply with sustainability requirements described in Division 01 General Requirements including, but not limited to, the following:

GREENGUARD Certification.

Recycled Content.

Carbon Reduction.

FSC Chain of Custody Certification.

TVOC (Total Volatile Organic Compounds).

Total Aldehydes.

Formaldehyde.

Life Cycle Analysis (LCA) or Durability.

Waste Management.

[Building Product Disclosure and Optimization – Environmental product declarations - environmental product declaration (EPD).]

[Building Product Disclosure and Optimization – Material Ingredients - Health Product Declaration (HPD).]

[LEED Rating System Requirements:

Potential Credits for Plastic Laminates:

MR Credit 4, Recycled Content.

MR Credit 5, Regional Materials.

MR Credit 7, Certified Wood.

IEQ Credit 4.5, Low-Emitting Materials.]

[Potential Credits for Adhesives:

IEQ Credit 3.2, Construction Indoor Air Quality.

IEQ Credit 4.1, Low Emitting Materials, Adhesives.

IEQ Credit 4.4, Low Emitting Materials, Adhesives, Composite wood and Agrifiber products.

IEQ Credit 4.5 Low Emitting Materials.]

[Enterprise Green Communities criteria for building certification.]

[Green Globes Sustainable Interiors criteria for building certification.]

Product Data: For recycled content, indicating postconsumer and preconsumer recycled content and cost.

Product Certificates: For materials manufactured within 500 miles (800 km) of Project, indicating location of material manufacturer and point of extraction, harvest, or recovery for each raw material. Include distance to Project and cost for each raw material.

Chain-of-Custody Certificates: For certified wood products. Include statement of costs.

Chain-of-Custody Qualification Data: For manufacturer and vendor.

Laboratory Test Reports: For adhesives, indicating compliance with requirements for low-emitting materials.

Laboratory test reports for IEQ 4.1 that indicate compliance with testing and product requirements of California Department of Public Health's "Standard Method for the Testing and Evaluation of Volatile Organic Chemical Emissions from Indoor Sources Using Environmental Chambers."

CLOSEOUT SUBMITTALS

Maintenance Data: Manufacturer’s written maintenance instructions.

Manufacturer warranties transferrable to Owner.

QUALITY ASSURANCE

Installer Qualifications: Company specializing in fabricating and installing decorative plastic laminate finished work with a minimum 3 years experience.

        - Fabricator Qualifications: Shop that employs skilled workers who custom fabricate products similar to those required for this Project and whose products have a record of successful in-service performance with a minimum 3 years experience. Shop must be approved by Formica Group to fabricate Intentek™ Laminate.  For list of qualified fabricators, visit formica.com/intentekfabricators.  Contact for special pricing.

Specifier Note:

If project size or complexity justifies cost for Mock-up(s) they can be specified here. Sample text below is for general projects that have full room mockups, such as hospital and hospitality projects.

Mock-Ups:

Build mockups to verify selections made under submittals and to demonstrate aesthetic effects, set quality for materials and construction, set quality standard for fabrication and installation.

Acceptable mock-ups may remain as part of the Work if undamaged at time of Substantial Completion.

Acceptable mock-ups shall be comparison standard for remaining Work.

DELIVERY, STORAGE, AND HANDLING

Package and ready materials according to manufacturer’s instructions.

Do not deliver components until Project is fully enclosed.

Store products inside building protected from light, heat and moisture and never store in contact with floor or outside wall surfaces. Do not expose to continuous direct sunlight.

Store horizontally

Store at 50-90 degrees F and 30% - 65% humidity.  Keep materials free from moisture, dust, and impact.  The material should never be stored in contact with the floor or an outside wall.  Store laminate sheets and backers as they have been shipped by the manufacturer.

Do not slide the back of the laminate against another surface.

Prior to fabrication, allow laminate sheet, backer, and substrate to acclimate for at least 48 hours at the same ambient conditions. The optimal conditions for acclimation are 50 – 90 degrees F (10-32 degrees C) and 30% – 65% humidity.

Provide protective coverings of suitable material. Take special precautions at corners.

Take precautions for wiring and electronics.

Power management should be coordinated with project owner or general contractor prior to installation.

PROJECT CONDITIONS

Coordinate sizes and locations of cut-outs and other related Work specified in other Sections to ensure that interior laminate construction can be supported and installed as indicated.

PRODUCTS

PERFORMANCE REQUIREMENTS

Fire-Test-Response Characteristics: Provide decorative plastic laminate with the following surface burning characteristics as determined by testing identical products per ASTM E 84 by UL or another testing and inspecting agency acceptable to authorities having jurisdiction:

Flame-Spread Index: 25 or less.

Smoke-Developed Index: 450 or less.

Source Limitations: Obtain decorative plastic laminate materials through one source from a single manufacturer.

MANUFACTURER

Manufacturer:

Basis of Design: Formica Corporation.

Call/email for special pricing information

Contacts:

www.formica.com/intentekinquiry

West: Harper Hendon, harper.hendon@formica.com , (214)-934-6659

East: Pat Weaver, pat.weaver@formica.com, (513)-604-2131

Central:  Michael Hertensteiner, michael.hertensteiner@formica.com (440)-668-7030

PLASTIC LAMINATE PRODUCTS

Formica Intentek™ Wireless Charging Surface

Specifier Note:

If working with Formica Group prior to specification, reference job # below for ease of contractor pricing requests.

[Reference Job # xxxx as provided by Formica Group team member]

Description:  decorative laminate with integrated wireless phone charging technology

WPC Qi Certified charging

The power is transferred from the Qi wireless transmitter to the Qi wireless receiver via magnetic resonant induction

Manufacturer electronic components package required

Placement of charging units and charging icons to be coordinated with selected fabricator and manufacturer.  For a list of approved fabricators, visit formica.com/intentekfabricators

Electronic module(s) installed in CNC milled recess beneath wireless charging laminate surface.

Laminate Grade:

Specifier Note:

Select grade(s) for the project. Some grades may come in certain specific colors and sizes.

[Grade TK, HGS - 0.090 Inches (2.3 mm).]

Laminate Color(s):

Specifier Note:

Select location of information or list the color selections for the project.

[As indicated on Drawings.]

[Match Architect’s samples.]

[As selected by Architect from manufacturer’s line of available colors.]

[Laminate Color (include here if not on Drawings).]

[Laminate Color (include here if not on Drawings).]

Laminate Finish:

Specifier Note:

Select location of finish information or Select finish selection(s) for the project.

[As indicated on Drawings.]

[Match Architect’s samples.]

[-58 Matte.]

Laminate Application(s):

Specifier Note:

Select application(s) for the project.

[Interior Finish Carpentry.]

[Plastic-Laminate-Faced Architectural Cabinets.]

[Interior Architectural Woodwork.]

[Manufactured Plastic-Laminate-Faced Casework.]

[Residential Casework.]

[Plastic-Laminate Laboratory Casework.]

[Plastic-Laminate-Clad Countertops.]

[Plastic-Laminate-Clad Case Goods.]

[Custom Laminate Furniture Applications]

LAMINATE ACCESSORY MATERIALS

Edge Banding:

Laminate Grade:

Specifier Note:

Select grade or type(s) for the project.

[Grade HGS.]

[PVC tape, 0.018-inch (0.460 mm) minimum thickness, matching laminate in color, pattern, and finish.]

[PVC edge banding, 0.12-inch (3 mm) thick, matching laminate in color, pattern, and finish.]

Self applied edge

See architectural drawings for edge detail.

Backing sheets (.090 inches   2.3mm):

Required by manufacturer to minimize lamination warpage.

Adhesives:

Bonding Laminate: Provide type recommended by manufacturer.

Bonding Edge Molding: Provide type recommended by manufacturer.

Intentek™ Electronics Module(s) and AC Adapter Power Cord(s).

LAMINATE FABRICATION

Specifier Note:

REFER to TECHNICAL BULLETIN FOR INTENTEK™ WIRELESS CHARGING SURFACE FOR FABRICATION AND INSTALLATION INSTRUCTIONS

Conform to Formica Corporation standard practices, procedures, conditions including preconditioning, panel balancing, material recommendations, machining, equipment and workmanship.

Router base should be clean and free of burrs and debris. Table saws should be clean, flat, and free of burrs.

Do not adhere laminates directly to plaster, gypsum board or concrete construction.

EXECUTION

INSTALLATION

General: Install decorative plastic laminate in accordance with manufacturer’s installation instructions, approved submittals and requirements of Intentek™ Wireless Charging Laminate by Formica Group.  Power management should be coordinated prior to installation.

Provide templates and rough-in measurements.

Accessory Materials: Install in accordance with manufacturer’s written installation instructions.

CLEANING AND PROTECTING

Cleaning:

Clean decorative plastic laminate surfaces and edge moldings in accordance with manufacturer’s instructions.

Protection:

Do not permit construction near unprotected surfaces.

END OF SECTION 06 05 13